#### How to Resetting Airbag Warning Light on 2010-2015 Nissan Leaf:

To turning off the Airbag Light on 2010, 2011, 2012, 2013, 2014, 2015 Nissan Leaf, conform to the following instructions:

- Turn the ignition key to position “ON” without starting the engine, If your vehicle has Smart key button, press the “Start” button twice without touching the brake pedal.

- Watch the airbag light closely. It will stay on for a few secs. and then as soon as it blinks off (the start of the flashing), turn the key off instantly.

- Wait for more than 3 seconds

- Repeat steps 1-3 two more times until you have done the cycle three times.

- Now turn the key on and watch the light. It will start blinking in a different, slower manner. This is diagnostic mode. You can watch this for a little while, no hurry.

- Now turn the key off again, wait for more than 3 seconds like in step 3 again, and turn back on.

- Turn the ignition off and then start the engine to verify the indicator has been reset